- Robust construction

- Advanced mobile weighing functionality

- Fast delivery times

- 0.1% of the load lifted

- Built-in 7.4Vdc/4Ah rechargeable Li-ion battery

- Printer optionally available

The HPT-X is a robust, basic hand pallet truck with integrated weighing system that brings mobile weighing of palletized goods in logistic warehouses or production sites a step closer.

Checking weights of incoming goods? Determine accurate shipping weights? Optimize your internal transport by saving time, space and costs with the HPT-X, the smart choice in mobile weighing.

General									Dimensions (in inch)

| A | Fork length | 48” |
| --- | --- | --- |
| B | Fork width | 7.1” |
| C | Fork height (min) | 3.25” |
| D | Fork height (max) | 7.9” |
|  | Lifting height | 4.8” |
| E | Width over forks | 27.2” |
| F | Height upper side | +/-32.5” |

Capacity			5,000 lb

Graduation			1 lb

Accuracy			0.1% of the load lifted

Display			LED, height 1.8”

Keypad			6 functions keys, standby on/off

on/off switch

Protection class      		loadcells and wiring IP67,

indicator IP65

Battery			7.4VDC/4Ah Li-ion

Charger			110 Volt

Autonomy			30 hours, low battery indication

Steering wheels		rubber

Load wheels			polyurethane, tandem

Colour				yellow/black, epoxy painted

Weight			235 lb

Standard functions

- Automatic and manual zero correction

- Gross/net weighing (tare)

- Totaling with sequence number

- On/off switch

- Auto shut-off

- Piece counting